PAPER SUMMARY

Quantum Reinforcement Learning for Volt-VAR Control

in Power Distribution Systems

Ding Lin, Jianhui Wang, Huan-Hsin Tseng, Tianqiao Zhao, Meng Yue, Shinjae Yoo

IEEE Open Access Journal of Power and Energy, Vol. 12, 2025  |  DOI: 10.1109/OAJPE.2025.3634993

# 1. Overview & Motivation

Volt-VAR Control (VVC) is essential in active power distribution networks — it optimizes voltage profiles and manages reactive power flow to ensure stable grid operation. Traditional VVC uses capacitor banks, voltage regulators, and load tap changers (LTCs). Controlling these devices simultaneously is a high-dimensional, complex optimization problem.

## Why Not Classical Deep Reinforcement Learning (DRL)?

- DRL can learn adaptive control policies from real-time data — promising for VVC.

- However, DRL requires large parameterized neural networks, raising concerns about model interpretability, overfitting risk, and heavy computational overhead.

- Over-parameterization becomes a serious bottleneck as network scale increases (e.g., 123-bus systems).

## The Quantum Solution

Variational Quantum Circuits (VQCs) are the quantum analogs of classical neural networks. They encode information into high-dimensional Hilbert spaces using superposition and entanglement, offering strong expressive power with far fewer trainable parameters. This paper proposes QE-SAC — a Quantum Enhanced Soft Actor-Critic algorithm — that achieves comparable or better performance than classical SAC using only ~1% of the parameters.

# 2. Problem Formulation (VVC as MDP)

The VVC problem is cast as a Markov Decision Process (MDP): M = (S, A, r, P, γ).

| Component | Description |
| --- | --- |
| State S | Voltages at all nodes, status of capacitors/regulators/batteries, real power losses, time step t |
| Action A | Adjustments to voltage regulators (tap positions), capacitor switching (on/off), battery discharge levels |
| Reward r | Negative of objective: penalizes voltage violations, control effort, and real power losses |
| Transition P | Computed by OpenDSS — a full 3-phase unbalanced AC power flow simulator |
| Discount γ | 0.99 — emphasizes long-horizon planning over a 24-hour control horizon (24 time steps) |

Table: MDP Components for the VVC Problem

## Optimization Objective

The agent minimizes a weighted combination of three loss functions over time:

- fvv (Voltage Violation): Counts how many node voltages fall outside [0.95, 1.05] per unit at each time step.

- fcl (Control Loss): Penalizes switching actions on capacitors, regulator tap changes, and battery discharging — discouraging unnecessary device operations.

- fpl (Power Loss): Ratio of real line losses to total system power — minimizing energy waste.

The reward at each step is r(st, at) = −[α·fvv + β·fcl + γ·fpl], with weights α, β, γ balancing competing objectives.

# 3. Proposed Algorithm: QE-SAC

## 3.1 Soft Actor-Critic (SAC) Foundation

SAC is the classical RL backbone. It uses maximum entropy reinforcement learning: the agent maximizes both expected return AND the entropy of its policy, promoting exploration and robustness. The objective is:

J(π) = E [ Σ γt (r(st,at) + α H(π(·|st))) ]

where α (temperature) controls the trade-off between reward and policy stochasticity. SAC uses two Q-networks (to reduce overestimation bias), one policy network, and an automatically tuned temperature α.

## 3.2 Variational Quantum Circuits (VQCs)

A VQC replaces the classical policy network's core layers. It has three stages:

| Stage | Method | Details |
| --- | --- | --- |
| Encoding | Angle Encoding (Ry gates) | Maps each latent feature s'i to a qubit rotation: ¦s'i⟩ = Ry(s'i)¦0⟩. Classical data → quantum state in Hilbert space. |
| Variational Layers | Rx rotation + CNOT entanglement | K variational layers, each with Rx(ςi) rotations on all qubits and CNOT gates on adjacent qubit pairs (linear nearest-neighbor topology). Parameters ς are learned. |
| Measurement | Pauli-Z expectation ⟨Z⟩ | Collapses quantum state to classical values: ⟨Z⟩ = ⟨ψ¦Z¦ψ⟩. Output passed to classical post-processing layers. |

Table: VQC Structure

Gradients of VQC parameters are computed using the parameter-shift rule: ∂⟨σz⟩/∂ςi = [⟨σz⟩(ςi + π/2) − ⟨σz⟩(ςi − π/2)] / 2

## 3.3 Hybrid CAE-VQC Architecture (Core Innovation)

The key challenge: current quantum hardware (NISQ era) supports very few qubits — far fewer than the state dimension of real distribution systems (e.g., 297 features for the 123-bus system). The solution is a three-stage hybrid pipeline:

| Stage | Component | Function |
| --- | --- | --- |
| 1 | Classical Autoencoder (CAE) | Encoder compresses high-dimensional state s ∈ ℝⁿ → compact latent vector s' ∈ ℝᵐ (m = number of qubits = 8). Decoder reconstructs s from s' during training only, ensuring information is preserved. Pre-trained offline on random rollouts, then periodically fine-tuned every C=500 gradient steps to track evolving state distributions. |
| 2 | Variational Quantum Circuit (VQC) | Processes the 8-dimensional latent vector s' on 8 qubits with 2 variational layers. Leverages quantum superposition and entanglement to extract rich policy features with minimal parameters (16 parameters per layer × 8 qubits). |
| 3 | Classical Post-Processing | Linear layer η: ℝᵐ → ℝ¦A¦ maps VQC output to per-device action dimensions. Softmax produces discrete probability distributions over actions for each control device. |

Figure 2 Summary: CAE-VQC Hybrid Policy Network

## 3.4 Domain-Specific Design for VVC

VVC has two specific challenges addressed by QE-SAC's architecture:

Factorized Per-Device Actor

With N heterogeneous control devices, the full joint action space A = A1 × A2 × ... × AN is combinatorially large and intractable to model directly. QE-SAC uses a factorized policy: π(at|st) = ∏ πi(a(i)t|st), outputting one categorical distribution per device from shared features. This enables device-specific policies while keeping complexity tractable.

Shared-Encoder Multi-Head Critic

Each of the two Q-networks shares a common state encoder and outputs N value vectors — one per device: Qφ(st) = [Q1φ(st,·), Q2φ(st,·), ..., QNφ(st,·)]. This allows shared feature learning while retaining device-level value estimation.

Co-Adaptive Autoencoder

Unlike prior QRL work that uses fixed encoders (e.g., PCA), QE-SAC continuously fine-tunes the autoencoder using a dedicated replay buffer BCAE (separate from the RL buffer). This keeps latent representations aligned with the non-stationary grid operating conditions. Stabilization strategies include: periodic updates every C steps, a smaller learning rate for the encoder, and reconstruction error monitoring with early stopping.

# 4. Experimental Setup

## 4.1 Benchmark Distribution Systems

All experiments use the PowerGym reinforcement learning environment, which wraps OpenDSS for realistic 3-phase unbalanced AC power flow simulation. Three IEEE benchmark feeders are tested:

| System | State Dim. | Capacitors | Regulators | Batteries | Cap. Status |
| --- | --- | --- | --- | --- | --- |
| IEEE 13-bus | 48 | 2 | 3 | 1 | 2 |
| IEEE 34-bus | 107 | 2 | 6 | 2 | 2 |
| IEEE 123-bus | 297 | 4 | 7 | 4 | 2 |

Table 1: Benchmark System Configurations

Each regulator has 33 tap positions; each battery has 33 discharge power levels; each capacitor is binary (on/off). The 24-hour horizon spans 24 discrete time steps.

## 4.2 Baseline Algorithms

| Algorithm | Description |
| --- | --- |
| Classical SAC | Standard discrete SAC, fully classical actor and critic. Two hidden layers of 256 units each. Best-performing classical architecture. Serves as the primary performance benchmark. |
| SAC-AE | Compact SAC matching QE-SAC's parameter scale. Uses the same co-adaptive autoencoder but replaces VQC with two hidden layers of only 8 units. Isolates quantum vs. compression benefits. |
| QC-SAC | Hybrid SAC with the same VQC as QE-SAC but uses a fixed PCA encoder (fitted offline, frozen during training). Tests importance of encoder adaptivity vs. quantum capacity. |
| QE-SAC (proposed) | Full method: co-adaptive CAE + 8-qubit 2-layer VQC + classical post-processing. Implemented in PennyLane with default.qubit backend. |

Table: Algorithm Baselines

## 4.3 Hyperparameters

| Parameter | QE-SAC / QC-SAC | SAC-AE | SAC |
| --- | --- | --- | --- |
| Hidden layers | — | 2 | 2 |
| Variational layers | 2 | — | — |
| Hidden units | — | 8 | 256 |
| Number of qubits | 8 | — | — |
| Learning rate | 1e-4 | 1e-4 | 1e-4 |
| Discount factor γ | 0.99 | 0.99 | 0.99 |
| Replay buffer size | 1,000,000 | 1,000,000 | 1,000,000 |
| Mini-batch size | 256 | 256 | 256 |
| Soft update factor ρ | 0.005 | 0.005 | 0.005 |
| CAE update freq. C | 500 | 500 | — |

Table 2: Hyperparameter Settings

# 5. Numerical Results

## 5.1 Ablation: Autoencoder Update Frequency

Three update frequencies were tested for the CAE fine-tuning interval C ∈ {100, 500, 1000} on the 123-bus system. All settings achieved stable convergence. C = 100 (most frequent) introduced slightly higher reward variance due to latent-space drift from rapid representation changes. C = 500 provided the best trade-off between adaptability and training stability and was adopted as the default for all subsequent experiments.

## 5.2 Performance Comparison (Table 3)

| Algorithm | Cum. Reward 13 | Cum. Reward 34 | Cum. Reward 123 | V. Viol. 13 | V. Viol. 34 | V. Viol. 123 | Time 13 (min) | Time 34 (min) | Time 123 (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QE-SAC | -5.39 | -5.15 | -9.53 | 0 | 0.04 | 0 | 22.5 | 20.8 | 28.1 |
| QC-SAC | -5.91 | -6.03 | -10.78 | 0.02 | 0.13 | 0.01 | 32.3 | 24.3 | 40.3 |
| SAC | -5.41 | -5.21 | -9.72 | 0.01 | 0.06 | 0 | 17.9 | 15.4 | 45.6 |
| SAC-AE | N/A* | N/A* | N/A* | N/A* | N/A* | N/A* | N/A* | N/A* | N/A* |

Table 3: Performance Comparison (Cumulative Rewards, Voltage Violations, Convergence Time) — * Cannot converge

## 5.3 Key Performance Findings

- QE-SAC achieved zero voltage violations on both the 13-bus and 123-bus systems — the primary safety metric.

- In the 34-bus system, QE-SAC reduced voltage violations to 0.04 vs. 0.06 for classical SAC — a 33% improvement.

- QE-SAC achieved comparable or better cumulative rewards vs. classical SAC across all three systems despite using ~1% of the parameters.

- SAC-AE (same parameter count, no quantum circuit) failed to converge on any system — confirming that quantum processing, not just parameter reduction, drives performance.

- QC-SAC (fixed PCA encoder + VQC) converged but with inferior performance vs. QE-SAC — highlighting the importance of the co-adaptive autoencoder.

- QE-SAC converged fastest in the largest 123-bus system (28.1 min vs. 45.6 min for SAC) — demonstrating quantum advantage at scale.

## 5.4 Parameter Efficiency (Table 4)

| Algorithm | 13-bus Parameters | 34-bus Parameters | 123-bus Parameters |
| --- | --- | --- | --- |
| SAC (classical) | 899,729 | 1,661,196 | 3,213,427 |
| SAC-AE | 4,440 | 12,492 | 41,259 |
| QE-SAC (proposed) | 4,872 | 13,212 | 42,329 |

Table 4: Learning Parameter Counts

QE-SAC uses ~100–200× fewer parameters than classical SAC while maintaining superior or equivalent performance. The parameter savings grow with system scale.

## 5.5 Convergence Speed by System

| System | QE-SAC Episodes to Convergence | QC-SAC Episodes | SAC Episodes |
| --- | --- | --- | --- |
| 13-bus | ~7,000 | ~9,000 | Slower |
| 34-bus | ~4,000 | ~5,000 | Slower |
| 123-bus | Fastest of all | ~5,000 | Slowest (45.6 min) |

Convergence Speed Summary

# 6. Robustness Under Quantum Noise

## Depolarizing Noise Model

Real quantum hardware suffers from noise — random qubit errors due to environmental interactions. The paper validates QE-SAC under a depolarizing noise channel: ε(ρ) = (1-λ)ρ + λ·I/2ⁿ, where λ is the depolarization probability. Tests used realistic error rates: 0.1% for single-qubit gates and 1% for two-qubit (CNOT) gates.

## Result

QE-SAC achieved nearly identical cumulative reward and voltage violation curves under both noisy and noise-free conditions on the 13-bus system. The algorithm is resilient to realistic quantum noise levels.

## Why Is QE-SAC Noise-Resilient?

- Shallow circuit depth (only 2 variational layers, 8 qubits) — fewer gates = fewer opportunities for noise accumulation.

- Small parameter count — less complexity exposed to random perturbations.

- Classical encoder enc(s) is entirely noise-free — quantum circuits only process the compact 8-dim latent representation.

- Classical post-processing layer partially mitigates degraded quantum measurement outputs before action selection.

# 7. Architecture Overview (Figure 1 & 2 Summary)

The QE-SAC system consists of two interacting components: the classical environment and the hybrid agent.

| Component | Role in QE-SAC |
| --- | --- |
| Classical Environment | IEEE distribution system simulated in OpenDSS. Provides 3-phase unbalanced AC power flow. Generates states, rewards, and next states. |
| Autoencoder (CAE) | Compresses high-dimensional state st (48/107/297-dim) → latent s't (8-dim). Pre-trained offline; fine-tuned every 500 steps using a separate CAE replay buffer. |
| VQC Policy Network | 8-qubit, 2-layer VQC processes latent state. Angle encoding → variational layers (Rx + CNOT) → Pauli-Z measurement. Output → linear layer → Softmax → per-device action probabilities. |
| Classical Value Networks | Two Q-networks (Qφ1, Qφ2) with multi-head architecture. Shared encoder + per-device heads. Standard fully-classical implementation. |
| Classical Optimizer | Updates all parameters: policy (ς via param-shift rule), Q-networks (φ via TD loss), temperature (α), and periodically the CAE (θenc, θdec). |

Figure 1 Summary: QE-SAC System Architecture

# 8. Algorithm Flow (Algorithm 1: QE-SAC)

| Step | Description |
| --- | --- |
| Init | Initialize encoder/decoder (θenc, θdec), VQC policy (ς), Q-networks (φ1, φ2), replay buffers B and BCAE. |
| Pre | Collect states via random policy. Pre-train CAE to minimize reconstruction loss (Eq. 17). Obtain initialized encoder. |
| 1 | Sample step: observe state st, select action at using current policy. Observe st+1, reward rt, done signal d. |
| 2 | Store (st, at, st+1, rt, d) in RL buffer B and (st, st+1) in CAE buffer BCAE. |
| 3 | Gradient step: sample mini-batch D. Update Q-networks by minimizing per-device TD loss (Eq. 31). |
| 4 | Update hybrid policy (VQC params ς) by minimizing per-device policy loss (Eq. 32) via parameter-shift gradients. |
| 5 | Update temperature α by minimizing entropy loss (Eq. 33) to match target entropy per device. |
| 6 | Soft update target Q-networks: φ'i = (1-ζ)φ'i + ζφi (ζ = 0.005). |
| 7 | Every C=500 steps: sample mini-batch from BCAE. Fine-tune θenc, θdec to minimize reconstruction loss (Eq. 18). |
| End | Repeat steps 1–7 until convergence. |

Algorithm 1: QE-SAC Training Procedure

# 9. Conclusions

This paper presents QE-SAC, the first quantum reinforcement learning algorithm specifically designed for multi-device Volt-VAR Control in realistic distribution systems. The three key innovations work together synergistically:

- Co-adaptive CAE: Bridges the dimensionality gap between large grid state spaces and quantum hardware qubit limits. Adapts to non-stationary grid conditions unlike prior fixed-encoder approaches.

- Hybrid CAE-VQC policy: Achieves strong representational power with ~1% of classical SAC parameters via quantum superposition and entanglement in a compact Hilbert space.

- Factorized per-device actor + multi-head critic: Handles the combinatorial action space of heterogeneous devices (capacitors, regulators, batteries) under AC power-flow coupling.

The results across IEEE 13-bus, 34-bus, and 123-bus systems confirm that QE-SAC:

- Achieves better or comparable voltage violation performance vs. classical SAC.

- Converges faster at scale (123-bus: 28.1 min vs. 45.6 min for SAC).

- Operates with ~100–200× fewer trainable parameters.

- Remains robust under realistic quantum hardware noise (0.1% / 1% gate error probabilities).

This work demonstrates the practical viability of quantum-enhanced reinforcement learning for complex, high-dimensional power system control under current NISQ hardware constraints.

# 10. Quick Reference: Key Equations

| Equation | Formula | Purpose |
| --- | --- | --- |
| Reward (Eq. 6) | r(st,at) = −[α·fvv + β·fcl + γ·fpl] | RL reward signal for VVC |
| SAC Objective (Eq. 7) | J(π) = E[Σ γt(r + α H(π¦st))] | Maximum entropy RL objective |
| Angle Encoding (Eq. 19) | ¦s'⟩ = ⊗ Ry(s'i)¦0⟩ | Maps latent features to qubits |
| VQC Layer (Eq. 20) | U(ςk) = ∏ CNOTi,j ⊗ Rx(ςki) | Variational + entanglement layer |
| Param-Shift (Eq. 25) | ∂⟨σz⟩/∂ςki = [⟨σz⟩(+π/2) − ⟨σz⟩(−π/2)] / 2 | Quantum gradient computation |
| CAE Pre-train (Eq. 17) | min (1/M) Σ ‖s − dec(enc(s))‖² | Offline autoencoder training |
| CAE Fine-tune (Eq. 18) | min (1/¦D¦) Σ ‖s − dec(enc(s))‖² | Online CAE adaptation |
| Factorized Policy (Eq. 27) | π(at¦st) = ∏ πiς(a(i)t¦st) | Per-device action distribution |
| Soft Update (Eq. 34) | φ'i = (1−ζ)φ'i + ζφi | Stable target network update |

Key Equations Summary

Summary prepared from: Ding Lin et al., 'Quantum Reinforcement Learning for Volt-VAR Control in Power Distribution Systems,' IEEE Open Access Journal of Power and Energy, Vol. 12, 2025.